# Object :

# Operating manual for Narishige puller (ID: ELP25) : microcapillaries stretching for microinjection

# Mailing list

BPMP, users of Electrophysiology technical platform

- Reference persons

# Anne-Aliénor Véry, Claire Corratgé-Faillie

# Health and safety

Risk of burning and pricking with pipettes. Use forceps remove pipettes out of the puller

- Operating mode

Adjust « N°1 heater » and « N°2 heater » values to control the thickness of capillaries tips (buttons 1 and 2)

The pulling is done in two steps:

* First step

-  Insert the capillary into the grooves provided for this purpose and tighten the screws 3 and 4 (after ascending block“4” in the upper position, see picture).
 - Move Up the block with the resistance 5

- Fold down the block with contact 6

- Fold the cover over the resistance 7

- Press start 8

 * Second Step :
When the resistance cools/gets off

- Move Down the block with resistance 5

- Open/Remove block 6

- Press "start" 8

# Ce document décrit le mode opératoire pour l’étirement de capillaires destinés aux micro-injections avec l’étireuse Narishige (ID: ELP25)

- Objet et domaine d’application

- Liste de diffusion et si nécessaire niveau de confidentialité

BPMP, utilisateurs de la plate-forme d’électrophysiologie

- Personnes référentes

# Anne-Aliénor Véry, Claire Corratgé-Faillie

- Hygiène et sécurité

Risques de brûlure et de piqûre avec les pipettes venant d'être étirées. Utiliser des pinces pour les sortir des mandrins de l'étireuse.

- Contenu du mode opératoire

Ajuster les paramètres « N°1 heater » et «  N°2 heater » en fonction de la finesse de la pointe désirée  (Boutons 1  et 2, voir schéma)

L’étirement se fait en 2 étapes :

*Première étape :

-positionner le capillaire dans les rainures prévues à cet effet et le maintenir en serrant les vis 3 et 4 après avoir remonté le bloc «4 » en position haute.

-Remonter le bloc comportant la résistance en position haute 5

- Rabattre le contact 6

- Rabattre le cache sur la résistance 7

- Appuyer sur start 8

*Deuxième étape

Lorsque la résistance s’éteint

-Descendre le bloc comportant la résistance en position basse 5

- ouvrir le contact  6

- appuyer sur « start » 8

1

2

3

4

5

6

7

8